SPECIFICATION FOR REFERENCE APPROVAL SIGNATURE. DATE:. CUSTOMER: 在 PART NO: REV: A 在 DESCRIPTION: PC 在 *PLEASE SIGN AND RETURN ONE COPY. *ALL PRODUCTION UNITS WILL BE BUILT ACCORDING TO THIS SPECIFICATIONS. PREPARED; CHECKED; APPROVED. DATE:; DATE:; DATE:. MODEL NO: NR-PSU5001 在 CATALOG NO: 在 AGENCY APPROVAL: 在 PRESENTED BY: 在 NO. P101228-01 在

WEB SITE : http://www.negorack.ru E-mail: sales@negorack.ru No.95 building, SanJiang industrial,HengLi Town, DongGuan, GuangDong, China Phone: +86 0755 33637353 QQ: 865001667

1.0 INPUT:	…………………………………………………………………3

1.1 VOLTAGE	3

1.2 FREQUENCY	3

1.3 CURRENT	3

1.4 INRUSH CURRENT	3

1.5 POWER EFFICIENCY	3

1.6 LEAKAGE CURRENT	3

1.7 POWER FACTOR ………………………………..….…………………………..3

1.8 ErP REQUIREMENT..………..…………………..….…………………………..3

2.0 OUTPUT:	4

2.1 REMOTE ON/OFF	4

2.2 HOLD-UP TIME	5

2.3 POWER GOOD DELAY	5

2.4 POWER FAIL DELAY	5

2.5 TURN-ON DELAY TIME	5

2.6 TRANSIENT OVERSHOOT	5

2.7 RISE TIME	5

3.0 PROTECTION:	5

3.1 OVER VOLTAGE PROTECTION	5

3.2 SHORT PROTECTION	6

3.3 OVER POWER PROTECTION	6

4.0 ENVIRONMENT:	6

4.1 OPERATING TEMP 	6

4.2 STORAGE TEMP.	6

4.3 OPERATING HUMIDITY 	6

4.4 STORAGE HUMIDITY……………………………………………….…………..	6

4.5 OPERATING ALTITUDE  	6

4.6 STORAGE ALTITUDE…………………………………………………………..	6

# 5.0 HI-POT:…………………………………………………………………..6

5.1 PRIMARY TO SECONDARY….………………………………………………. 	6

# 6.0 SAFETY AND EMC REQUIREMENTS:	……………………………….6

6.1 CONDUTED EMI…………………………………..………………………………6

6.2 SAFETY STANDARDS ………………………….….……………………………6

6.3 HARMONIC…………… ……………………………..……………………………6

# 7.0 MTBF at 250C (demonstrated)……………………………………….7

8.0 DIMENSIONS…………………………………………………………...7

# 1.0 INPUT:

## 1.1 VOLTAGE

| ##### MINIMUM | ##### NOMINAL | ##### MAXIMUM | ##### UNITS |
| --- | --- | --- | --- |
| ##### 207 | ##### 230 | ##### 253 | ##### Vrms |

## 1.2 FREQUENCY

47Hz ~ 63Hz

## 1.3 CURRENT

230Vac / 4.0A max.

## 1.4 INRUSH CURRENT

100A max. when AC input 230Vac and 250C cold start.

## 1.5 POWER EFFICIENCY

80% minimum under the load conditions defined in below table and 230Vac input.

Loading Table for Efficiency Measurements

| 500W(loading shown in Amps) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Loading | +12V1 | +12V2 | +5V | +3.3V | -12V | +5Vsb |
| Typical | 8.18 | 7.16 | 6.65 | 7.70 | 0.14 | 1.13 |

## 1.6 LEAKAGE CURRENT

3.5mA max.

## 1.7 POWER FACTOR

PF > 0.9 at 230Vac input

## 1.8 ErP REQUIREMENT

Meet ErP 2010 requirement

# 2.0 OUTPUT:

| ##### Voltage |  | +5V | +3.3V | +12V1 | +12V2 | -12V | +5Vsb |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ＊ | ###### Max load | 19.0A | 22.0A | 24.0A | 21.0A | 0.3A | 2.5A |
|  | Min load | 0.5A | 0.3A | 0.5A | 0.5A | 0.0A | 0.0A |
|  | Peak load | -- | -- | 27.0A | 23.0A | -- | 3.0A |
| ＊ | Combined power | 130W |  | 408W |  | -- | -- |
| ＊ | Regulation | +5,-5% | +5,-5% | +5,-5% | +5,-5% | +10,-10% | +5,-5% |
| ＊ | Ripple & Noise | 50mV | 50mV | 120mV | 120mV | 120mV | 50mV |

＊ The continuous total output power is 500W max.

- The combined power of +5V and +3.3V is 130W max.

- The combined power of +12V1 and +12V2 is 408W max

- Peak currents may last up to 12 seconds with not more than one occurrence per minute.

＊ Add 0.1uF and 10uF capacitors across output terminal during ripple & noise test.

＊ LOAD REGULATION TEST TABLE:

|  | +3.3V | +5V | +12V1 | +12V2 | -12V | +5Vsb |
| --- | --- | --- | --- | --- | --- | --- |
| LOAD1 | 0.3A | 0.5A | 0.5A | 0.5A | 0A | 0A |
| LOAD2 | 8.0A | 7.0A | 8.0A | 7.0A | 0.15A | 1.0A |
| LOAD3 | 10.5A | 19.0A | 16.5A | 13.0A | 0.3A | 2.5A |
| LOAD4 | 22.0A | 11.5A | 16.5A | 13.0A | 0.3A | 2.5A |
| LOAD5 | 6.5A | 11.0A | 19.0A | 15.0A | 0.3A | 2.5A |

## 2.1 REMOTE ON/OFF

TTL High/PS-OFF; TTL Low/PS-ON

Vil=0.8Vmax, Iil=-1.6mAmax @Vin=0.4V

Vih=2.0Vmin @Iin=-200uA, Vih=5.25Vmax @open ckt.

## 2.2 HOLD-UP TIME

10msec (minimum) at 80% of full load and 230Vac input.

## 2.3 POWER GOOD DELAY

100-500 msec.

## 2.4 POWER FAIL DELAY

1 msec.

# 2.5 TURN-ON DELAY TIME

2000 msec max.At Nominal Line Full Load.

## 2.6 TRANSIENT OVERSHOOT

DC output transient step sizes as below table:

| Output voltage | +5V | +3.3V | +12V1 | +12V2 |
| --- | --- | --- | --- | --- |
| Max. step size | 30% | 30% | 40% | 40% |

Load-changing repetition rate of 10m seconds.

Load slew rated 1.0A/uS and capacitive load as below :

| +5V | +3.3V | +12V1 | +12V2 | -12V | +5Vsb |
| --- | --- | --- | --- | --- | --- |
| 6000uF | 6000uF | 5000uF | 3000uF | 470uF | 470uF |

## 2.7 RISE TIME

20ms max at full load.

# 3.0 PROTECTION:

When OVP ,OPP or short protection is triggered, the main outputs will be latched off. The main outputs can be reset by cycling the DC remote on/off or AC power. +5Vsb output is auto recovery when fault condition removed.

3.1 OVER VOLTAGE PROTECTION

+3.3V output     4.5 Vmax.

+5.0V output     7.0 Vmax.

+12.0V output   15.6 Vmax.

## 3.2 SHORT PROTECTION

All output to GND.

## 3.3  OVER POWER PROTECTION

Foldback at 110%~150% over peak load

4.0 ENVIRONMENT:

## 4.1 OPERATING TEMP.            0 oC to  +40 oC

## 4.2 STORAGE TEMP.               -20 oC to  +70 oC

## 4.3 OPERATING HUMIDITY     20% to 90%,non-condensing

## 4.4 STORAGE HUMIDITY         5% to 95%, non-condensing

## 4.5 OPERATING ALTITUDE      0 to 10,000 feet

## 4.6 STORAGE ALTITUDE         0 to  50,000 feet

5.0 HI-POT:

## 5.1 PRIMARY TO SECONDARY

1800Vac for 1 minute

# 6.0 SAFETY AND EMC REQUIREMENTS

## 6.1 CONDUCTED EMI

1. MEET CISPR 22： Class B

## 6.2 SAFETY STANDARDS

1. MEET TUV (EN60950)

2. MEET CB (IEC 950 )

- 4. MEET CE

5. MEET CCC

## 6.3 HARMONIC

MEET IEC1000-3-2,Class D

7.0 MTBF at 25C(demonstrated)

80K hrs minimum

8.0 DIMENSIONS

WxLxH=150x140x86mm

GPAxP   80*25mm FAN x1 IN FRONT

GPAxS   120*25mm FAN x1 ON CASE TOP